4.2 Leaf Cell Description

- Delay

Functionality

module shift_register_16_tb;

reg Din, clk, rst;

wire [15:0] Q;

shift_register_16 uut (Q, Din, clk, rst);

always #5 clk = ~clk;

initial

begin

$monitor("Time=%0t | Din=%b | Q(Binary)=%b",$time, Din, Q);

clk = 0; rst = 1; Din = 0; #10;

rst = 0;

Din = 1; #10;

Din = 1; #10;

Din = 0; #10;

Din = 0; #10;

Din = 0; #10;

Din = 0; #10;

$finish;

end

endmodule

- Optimization used

Shift register implementation to reduce control complexity

Register sharing to save hardware

Efficient clocking to reduce power consumption

- Multiplier

Functionality

module multiplier_tb;

reg signed [15:0] A, B;

wire signed [31:0] P;

multiplier_bit uut (

.A(A),

.B(B),

.P(P)

);

initial

begin

$monitor("Time=%0t | A=%b B=%b | P=%b",$time, A, B, P);

A = 1; B = 1; #10;

A = 2; B = 3; #10;

A = 3; B = 4; #10;

A = 5; B = 2; #10;

A = 6; B = 3; #10;

A = 7; B = 2; #10;

A = 8; B = 3; #10;

A = 9; B = 2; #10;

$finish;

end

endmodule

- Optimization used

Booth’s Algorithm to reduce number of partial products

(Booth’s Algorithm is a technique used in signed binary multiplication to make multiplication)

Use of pipelining to increase speed

Utilization of DSP blocks (in FPGA) for efficient multiplication

- Adder

Functionality

module adder_16bit_tb;

reg [15:0] A;

reg [15:0] B;

reg Cin;

wire [15:0] Sum;

wire Carry;

adder_16bit uut (

.A(A),

.B(B),

.Cin(Cin),

.Sum(Sum),

.Carry(Carry)

);

initial

begin

$monitor("Time=%0t | A=%0d B=%0d Cin=%b | Sum=%b Carry=%b",

$time, A, B, Cin, Sum, Carry);

A = 1;  B = 1;  Cin = 0; #10;

A = 2;  B = 3;  Cin = 0; #10;

A = 4;  B = 5;  Cin = 0; #10;

A = 6;  B = 2;  Cin = 0; #10;

A = 7;  B = 1;  Cin = 1; #10;

A = 8;  B = 1;  Cin = 0; #10;

A = 9;  B = 9;  Cin = 0; #10;

$finish;

end

endmodule

- Optimization used

4-bit Adder for faster operation

Reduced propagation delay

Bit-width optimization to minimize hardware usage

4.3 Leaf Cell Simulation

- Delay

- Input test cases

The reset signal (rst = 1) clears the 16-bit shift register, making all output bits zero. After the reset is removed, the input Din is set to 1 for two clock cycles, so two 1s are inserted into the register. Later, Din is changed to 0, and with every clock pulse the stored bits shift one position to the left while 0 enters from the right side. Because of this shifting operation, the pair of 1s moves left across the register, producing the output sequence 0001, 0003, 0006, 000C, 0018, and 0030, which shows the operation of a 16-bit left shift register.

- Output verification

| Time (ns) | clk ↑ | Din | rst | Q[15:0] (Hex) | Q[15:0] (Binary) |
| --- | --- | --- | --- | --- | --- |
| 0 | Reset | 0 | 1 | 0000 | 0000 0000 0000 0000 |
| 10 | ↑ | 1 | 0 | 0001 | 0000 0000 0000 0001 |
| 20 | ↑ | 1 | 0 | 0003 | 0000 0000 0000 0011 |
| 30 | ↑ | 0 | 0 | 0006 | 0000 0000 0000 0110 |
| 40 | ↑ | 0 | 0 | 000C | 0000 0000 0000 1100 |
| 50 | ↑ | 0 | 0 | 0018 | 0000 0000 0001 1000 |
| 60 | ↑ | 0 | 0 | 0030 | 0000 0000 0011 0000 |

- Multiplier

- Input test cases

The input test cases for the 16-bit multiplier circuit were applied by changing the values of inputs A and B at different time intervals. Initially, at time 0, both inputs were given as 1, producing an output product of 1. After every 10 time units, new input combinations were applied to verify the multiplication operation. The test cases included values such as A=2 and B=3 giving output 6, A=3 and B=4 giving output 12, A=5 and B=2 giving output 10, and so on. The inputs were provided in both decimal and 16-bit binary format, while the output product was observed in 32-bit binary format. These test cases help confirm that the multiplier correctly performs multiplication for different combinations of input values and produces the expected output product.

- Output verification

| Time | A (Decimal) | B (Decimal) | P = A×B (Decimal) | P (Binary) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 0000_0000_0000_0000_0000_0000_0000_0001 |
| 10 | 2 | 3 | 6 | 0000_0000_0000_0000_0000_0000_0000_0110 |
| 20 | 3 | 4 | 12 | 0000_0000_0000_0000_0000_0000_0000_1100 |
| 30 | 5 | 2 | 10 | 0000_0000_0000_0000_0000_0000_0000_1010 |  |
| 40 | 6 | 3 | 18 | 0000_0000_0000_0000_0000_0000_0001_0010 |  |
| 50 | 7 | 2 | 14 | 0000_0000_0000_0000_0000_0000_0000_1110 |  |
| 60 | 8 | 3 | 24 | 0000_0000_0000_0000_0000_0000_0001_1000 |  |
| 70 | 9 | 2 | 18 | 0000_0000_0000_0000_0000_0000_0001_0010 |  |

- Adder

- Input test cases

The test cases for the 16-bit adder use small decimal inputs to verify basic addition functionality and carry behavior. Inputs range from simple values like (1, 1), (2, 3), (4, 5), and (6, 2) to slightly larger cases such as (7, 1 with Cin=1), (8, 1), and (9, 9), all chosen within 1 to 9 for easy understanding. Each test case applies different combinations of A, B, and Cin to check correct sum generation in binary form and to ensure proper carry output. These cases mainly verify normal addition situation where results remain within 16-bit limits, helping confirm correct operation of sum and carry logic in the adder design.

- Output verification

| A (Decimal) | B (Decimal) | Cin | Sum (Decimal) | Carry |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 2 | 0 |
| 2 | 3 | 0 | 5 | 0 |
| 4 | 5 | 0 | 9 | 0 |
| 6 | 2 | 0 | 8 | 0 |
| 7 | 1 | 1 | 9 | 0 |
| 8 | 1 | 0 | 9 | 0 |
| 9 | 9 | 0 | 18 | 0 |